Patent Claims

- Arresting arrangement for swivel castor’s controlled motion, comprising: a cover; a spring-action element and an arresting element, the cover (1) is with an opening (4), housing a bushing (5) abutted to which is a carrying arm (6) of a swivel castor (7), a disk (10) is bonded to the bushing (5), between the cover (1) and the disk (10) there is a cushion (11) with a nipple (12), feeding fluid to it; its upper end juts out above the cover (1), wherein below the disk (10) there is a spring-action element (2) to which an arresting element (3) is bonded, the spring-action element (2) and the arresting element (3) are bonded to the flange (13) of the cover (1); they are in an axial motion to the carrying arm (6) and the swivel castor (7).

- An arresting arrangement per Claim 1, in which the spring-action element (2) features a spring-like profiled disk.

- An arresting arrangement per Claim 1, in which the arresting element (3) features an arresting disk, the nether surface of which provides for the existence of a high level coefficient of friction between it and contact surfaces of swivel castors.

- An arresting arrangement per Claim 1, 2 or 3, in which the periphery of the supporting disk (10) is with projections (14), locking up with cuts in the flange (13) of the cover (1).